Report date: 17th January 2011

Table: Average water quality for Kampala during the period of 10th - 16th January 11

| Parameter | Monitoring points |  |  |  |  |  |  |  | National Standard For potable water quality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Raw water | Treated Water Water Works | Water in Supply |  |  |  |  |  |
|  |  |  |  | Rubaga Division | Nakawa Division | Makindye Division | Kampala Central | Kawempe Division |  |
| pH | … | 7.3 | 7.0 | 7.2 | 7.1 | 7.3 | 6.9 | 7.0 | 6.5- 8.5 |
| Turbidity | NTU | 18 | 0.90 | 0.52 | 0.38 | 0.28 | 0.16 | 0.32 | 5.00 |
| Colour | PTU | 130 | 6 | 2 | 3 | 5 | 1 | 5 | 15 |
| Free chlorine | mg/l | NA | 1.00 | 0.30 | 0.35 | 0.35 | 0.30 | 0.35 | 0.2-0.5 |
| E-Coli | CFU/100 ml | 40 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Faecal -Streptococi | CFU/ 100 ml | 32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

NA: Not applicable