Report date: 05th October 2009

Table: Average water quality for Kampala during the period of 28th Sept- 04th October 2009

| Parameter | Monitoring points |  |  |  |  |  |  |  | National Standard For potable water quality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Raw water | Treated Water | Water in Supply |  |  |  |  |  |
|  |  |  |  | Rubaga Division | Nakawa Division | Makindye Division | Kampala Central | Kawempe Division |  |
| pH | … | 8.40 | 7.20 | 7.06 | 7.11 | 7.07 | 7.06 | 7.10 | 6.5- 8.5 |
| Turbidity | NTU | 18.9 | 0.80 | 0.48 | 0.38 | 0.43 | 0.38 | 0.65 |  5.00 |
| Colour | PTU | 138 | 6 | 5 | 7 | 10 | 2 | 11 |  15 |
| Free chlorine | mg/l | - | 1.40 | 0.35 | 0.30 | 0.40 | 0.35 | 0.30 | 0.20 |
| E-Coli | CFU/100 ml | 48 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Faecal -Strepstococi | CFU/ 100 ml | 34 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

NA: Not applicable