Report date: 1st March 2010

Table: Average water quality for Kampala during the period of 22nd - 28th February 10

| Parameter | Monitoring points |  |  |  |  |  |  |  | National Standard For potable water quality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Raw water | Treated Water | Water in Supply |  |  |  |  |  |
|  |  |  |  | Rubaga Division | Nakawa Division | Makindye Division | Kampala Central | Kawempe Division |  |
| pH | … | 7.40 | 7.00 | 6.98 | 7.01 | 6.88 | 7.05 | 7.15 | 6.5- 8.5 |
| Turbidity | NTU | 29.5 | 0.78 | 0.34 | 0.28 | 0.34 | 0.26 | 0.52 |  5.00 |
| Colour | PTU | 206 | 0 | 7 | 1 | 1 | 1 | 8 |  15 |
| Free chlorine | mg/l | - | 1.00 | 0.25 | 0.25 | 0.25 | 0.25 | 0.35 | 0.20 |
| E-Coli | CFU/100 ml | 42 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Faecal -Strepstococi | CFU/ 100 ml | 32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

NA: Not applicable